Stand Plan Submission Procedures

Complex Structures

Definition

A Complex Structure is any form of construction of any height, which would normally be designed by an engineer and/or has, through a risk assessment, been found to provide a significant risk. Structures ground supported over 4m in height, multi-story stands and all suspended items are always considered to be Complex Structures unless demonstrably simple and possessing no significant risk.

Examples of complex structures:

- Any structure, regardless of its height, which requires structural calculations

- Multi-storey stands

- Any part of a stand or exhibit which exceeds 4 metres in height

- Suspended structures (does not include banners) e.g. lighting rigs, bespoke custom wooden built

- Sound/lighting towers

- Temporary tiered seating

- Platforms and stages 0.6 meters in height or over

- Platforms and stages of any height for public use (not including stand floor flats and platforms)

- LED wall of any height set on top of a platform

Your Event Manager and Excel’s Health & Safety Service Department can assist you in identifying Complex Structures.

Submission Procedure

Licensees are responsible for submitting full details of all complex structures, no later than 28 days prior to the first day of the licence period, to their Excel Event Manager and to the engineers.

The following details are required:

- Detailed, scaled structural drawings (including staircases, plan views and elevations) with dimensions.

- Calculations, Specifications and each showing Event name and stand number.

- For each Complex Structure the Licensees are required to send 3 complete sets of the above in English. Items should not be sent piecemeal.

- Risk Assessment & Method Statement from construction company.

- Any Complex Structure that is modified after its initial submission to The Company must be re-submitted for certification by the organiser detailing all modifications.

Sign Off

All Complex Structures must be certificated by a suitably qualified and experienced person; generally, a Chartered Engineer with adequate professional indemnity cover, to confirm that the design is safe for its purpose. This person should:

- Arrange that requirements for safety are met.

- Manage the procurement, including provision by the supplier of construction drawings accompanied by calculations, design loads and relevant test results.

- Confirm the final overall design is safe for its purpose then produce a certificate to build.

Once installed the Licensees engineer must provide on-site inspection certificates for all Complex Structures to the Excel Event Management Department.

Space only stands and Features

Definition

Any stand or structure that is neither complex nor shell scheme system (such as Octanorm). LED wall not set on a platform and below 4m high.

Submission Procedure

Organisers are not required to submit non-complex space only stand plans. However, if there are numerous instances of non-compliant stands being built during your event, identified on site (based on the rules detailed in the AEV eGuide document), we will insist on the submission of all space only stand plans on your future events.

Sign Off

All space only stands will be subject to a visual sign off by a Structural Engineer.

Platforms below 600mm (single section platforms), with no access to public also no back wall or set do not require an engineer’s sign off.

Non-compliance

Any non-compliant issues that are found during your event will be reported directly to the organising team on-site along with the nominated health and safety team and these would be expected to be dealt with accordingly.

Frequent Complex stands / space only, non-compliant issues that we are keen to avoid are:

- Thickness of glazing (the minimum thickness of a pane, based on its horizontal and vertical dimensions, can be found in the 2026 AEV eGuide, ‘Glazing’, sub-section ‘Construction Materials’, section ‘Stand Construction’). Please note that if either the horizontal or vertical dimensions of any band are exceeded, the pane must be judged on the next band up; therefore, it is not a measure of square area

- The absence of Vision panels in storeroom doors

- Non-compliant stepped access, the 2026 AEV eGuide, ‘Stepped Access’

Shell System Construction

The shell system contractor must confirm in writing that they have carried out an inspection of all shell scheme stands, and they have been constructed using the correct material, in accordance with manufactures specification and are fit for purpose.

This sign off should be submitted to the venue event manager prior to the event opening to the public.

Excel London’s Health & Safety Services Team will inspect all stands, structures and features; to check they are fully compliant with the regulations set out in the AEV eGuide.